023-0132E	EXPLANATION 13

### RAW MATERIALS AND FUELS

In the covenant and its appendices rules are set for the accounting of fuels. These rules must be specified, in case the distinction between raw material and fuel is not clear.  The basic rule always remains that the energetic comparison must be pure and functional; flows or sub- processes belonging to the process, are included in the benchmark; what is not part of the process will probably be left outside the benchmark.

To explain this complex matter, a number of types are given as example.  In practice particular types may be combined or require an ad hoc solution.  An alternative approach for existing benchmark systems may be acceptable.

Type 1: A process with raw material and product without energy content

This is the most common type.  Neither the raw material nor the product contain energy.

The specific energy consumption will benchmarked, as     A / P.

Type 2: A process with by-product and/or waste.

Raw material and product don’t contain energy, but besides the main product which is benchmarked, there is also a by-product that probably is not present in every process of the benchmark.  Also waste may occur.

The material balance is closed, this means the sum of main product, by-product and waste streams equals the raw material input; any vaporized moisture or solvent must be included in the waste stream.

In this case, only the main product is benchmarked.   If energy (n) is required to valorize the by-product, this energy amount is excluded from the benchmark and therefore subtracted from the energy supply.

The benchmarked energy consumption is     (A-n) / P

The more waste is produced, the more unfavourable the benchmarking result will be.

If there are several main products, the benchmarking method must be concentrated to a typical mix.  Deviations from this mix may be corrected.

Type 3: A process with secondary energy output.

Several processes have a secondary energy output, as heat (usually in the form of steam) or electricity.  If this energy is used elsewhere, it is subtracted from the energy consumption when benchmarking the process, but added as input to the consuming process.

This subtraction must be calculated back to primary energy, since benchmarks are always on the level of primary energy.  In the covenant this conversion is normally done with an efficiency of 0,9 for heat and 0,4 for electricity.

The benchmarked energy consumption is       (A-B’) / P

where B’ represents  primary energy, calculated from B.

Type 4: An exothermic process

A lot of chemical processes are exothermic.  In the figure there is no energy input at all, but only energy output.

The benchmarked consumption is a negative figure, being      – B’ / P

where B’ represents the primary energy calculated from B; the subtraction is conditional to a use elsewhere in the plant, or outside.

Type 5: A process with a raw material with energy content and a by-product with energy content.

In this type the material balance is closed, the energy input of the raw material is completely converted into the end product and the by-product.  The raw material is not used as energy.

Hydrogen is a well-known example of a by-product with energy content.  Also several hydrocarbons may be examples in well-defined processes.

As in type 2, the possible extra energy consumption (n) required to valorize the by-product,  is excluded from the benchmark and therefore subtracted from the energy input (A).

The by-product can be used as a raw material or as a fuel, but that can not be controlled by the plant manager.  Therefore the energy value of the by-product must be excluded from the benchmark.

The benchmarked consumption is      (A-n) / P

Remark regarding national balances:

If later on the energy content of the by-product is used in another process, it must be considered there as an energy input and included in the national energy and CO2 balances.

Type 6: A process with export of a secondary fuel

A secondary fuel is a stream which originates from primary energy, for example blast furnace gas.

As the plant manager can decide if the secondary fuel will be used, it must be included in the benchmark.  If the secondary energy is used elsewhere, it is subtracted from the primary energy consumption.

The benchmarked consumption is	(A-Y)  /  P

Type 7: A process with a raw material with energy content and by-product which is used as fuel

If the processing plant produces a by-product, which is used as fuel in the process, the split can be virtually placed in the beginning of  the block diagram.

Well known examples are oil refineries, crackers, ethylene oxide, paper and wood industry.

Essential is that the material balance shows a deficit, because part of the supplied raw material was used as fuel in the process.  In the block diagram the raw material input is split in a raw material part and a part primary energy.  The latter (C) is added to the energy supply (A).  An output of secondary energy (B) is also still possible.

The benchmarked net consumption is      (A+C-B’) / P

Remark regarding the national balances:

The amount of energy from the raw material C must also be included in the national energy balance! Also the corresponding CO2 emission must be included in the CO2 balance.  If in its further life cycle the product is completely or partially used as energy, it must be counted as energy supply then and included in the energy and CO2 balances.

Type 8: A process with raw material and waste, both with energy content

The waste has an energy value, but the decision whether the energy will be valorized or not is out of the control of the plant manager.

The benchmarked consumption is         A / P

If  afterwards an extra process is necessary to upgrade the waste (into energy or not), it can’t be considered as waste any more, but as a by-product.  Then a subtraction as in type 2 and 5 will be applicable.

A special case belonging to this type are emergency flares in the chemical industry: the flared gasses must be considered as waste.  The auxiliary burners must be considered as energy consumers: if they are not included in A, their consumption must not be forgotten in the final balance of the company!

Remark:

Incineration of waste is not included in the national energy balance, but it must be included in the CO2 balance under the heading “waste incineration”.

Type 9: A process with raw material with energy content and a side-stream with energy content which is valorized

Some side-streams can’t be considered as waste, for example solvents.  They can (or must) be burned afterwards and in general their energy content will be recovered.  This amount must be included in the benchmark.  Therefore these streams are split from the raw material input and as equivalent primary fuel value added to the energy supply.  On the other hand the used energy (Z) is calculated back to primary energy Z’ and subtracted.

The benchmarked consumption is            (A+C-Z’) / P

Remark:

Also here the energy flow C must be included in the national energy and CO2 balances.

Type 10: A process with a raw material with energy content, from which waste is combusted internally

This exceptional case applies when :

- it concerns waste originating from the raw material

- it can’t be included in the energy balance

- the combustion is exceptional and can’t be found in benchmark data

- the combustion can’t be included in the benchmark

- the combustion should be included in the national CO2 balance

In this case the combustion must be excluded from the benchmark.  The benchmarked consumption is           A / P

It is obvious that this rule applies for each installation of the benchmark.